Biostratigraphic Analysis Micropaleontology Statoil, Desoto Canyon Block 231, OCS-G-33780 #1 [Perseus] Interval: 7976.5’ – 23085’ Prepared for Statoil Gulf Services LLC by Russell E. Hinote 12 February 2015 Ellington ALS Oil & Gas 1414 Lumpkin Road Houston, TX 77043 USA T: +1 713 956 2838 F: +1 713 481 5333 www.ellingtongeologic.com

Biostratigraphic Analysis

Micropaleontology

Statoil, Desoto Canyon Block 231, OCS-G-33780 #1

[Perseus]

1. INTRODUCTION

This report provides a biostratigraphic analysis of cuttings samples within the depth range of 7976.5 feet to 23085 feet in the subject well. In total, 525 washed residues were prepared from cuttings samples by ALS Ellington for microfossil analysis. Raw cuttings that were sampled from the wellbore at 10-foot intervals were composited for preparation of residues as follows: 30-foot intervals from 8000’-18020’, 20-foot intervals from 18020’-19200’, and 30-foot intervals from 19200’-2308’. The first and last samples in the set were not composited with others.

All residues were examined to determine occurrence and relative abundances of all foraminifera, ostracods, and any other identifiable microfossils, along with general lithologic characteristics of the residues, in order to determine ages and paleoenvironments. Significant biostratigraphic events are presented below in Section 2. Age interpretations are based primarily on the correlation of occurrences of key forams and ostracods in samples with their accepted age ranges. Paleoenvironmental interpretations are summarized in Section 3, and are based on the occurrence of environmentally diagnostic fossil groups combined with dominant lithologic characteristics. Definitions and descriptions for the paleoenvironmental zonation used in this report are shown in Appendix 1. Figures 1 and 2 in the appendix show the environmental codes used in this report for carbonate and terrigenous paleoenvironmental interpretations.

A key explaining relative abundance designations for fossil taxa used in this report and in Addendum 1 is found below in this document (Appendix 2).  Addendum 1 included as a companion to this document is a digital Excel file containing 2 data spreadsheets. The first spreadsheet, Appendix 3, provides the relative abundances of all observed fossil taxa per sample. Appendix 4, the second spreadsheet, contains a detailed sample-by-sample listing of lithologic observations and resulting paleoenvironmental interpretations. It should be noted that the lithologic descriptions of samples relate to the processed sample residues only, and will differ from any descriptions of bulk cuttings made outside this study, for the reason that a substantial amount of rock matrix is washed away during processing.

ATTACHMENT:

Addendum 1: ALS-EA Statoil DC Blk 231 #1 Perseus - Addendum 1 Datasheets 2-12-2015

2. BIOSTRATIGRAPHIC INTERPRETATIONS

Significant biostratigraphic events are presented below. Age interpretations are based primarily on the correlation of occurrences of key forams and ostracods in samples with their accepted age ranges. The older units are sparsely fossiliferous to unfossiliferous, and so unit identifications and ages are based mainly on general correlations of characteristic lithologies.

Abbreviations:

LAD = Last Appearance Datum (first downhole occurrence)

FAD = First Appearance Datum (last downhole occurrence)

(F) = Foram, (O) = Ostracod

| Sample Depth | Biostratigraphic Events | Age [Formational Unit] |
| --- | --- | --- |
| 7976.5’ – 8000’ | First sample examined. Key fossils present: Dictyoconus walnutensis (F), Cytheridea brevis (O), Cytheridea goodlandensis (O) | No younger than Middle Albian [In Fredericksburg] |
| 9140’ – 9170’ | FAD Dictyoconus walnutensis (F) |  |
| 9230’ – 9260’ | LAD Lituola camerata (F) | Middle Albian [Lower Fredericksburg] |
| 9710’ – 9740’ | LAD Eocytheropteron trinitiensis (O) | Early Middle Albian [Upper Trinity] |
| 10160’ – 10190’ | FAD Cytheridea goodlandensis (O) FAD Eocytheropteron trinitiensis (O) FAD Lituola camerata (F) | [possible base Upper Trinity] |
| 10250’ – 10280’ | LAD Dictyoconus sunnilandensis (F) - rare | Early Albian [Lower Trinity] |
| 10340’ – 10370’ | LAD Orbitolina texana (F) | Early Albian [Lower Trinity] |
| 12500’ – 12530’ | LAD Choffatella decipiens (F) FAD Orbitolina texana (F) | Early Aptian-Barremian [Coahuilan/Sligo equivalent] |
| 13220’ – 13250’ | LAD Pseudocyclammina lituus (F) | No younger than Barremian |
| 15290’ – 15320’ | LAD Lenticulina busnardoi (F) | Early Hauterivian or older |
| 15440’ – 15470’ | LAD Eponides caracolla (F), Lenticulina nodosa (F) | No younger than Valanginian [Upper Cotton Valley] |
| 15770’ – 15800’ | LAD Lenticulina ouachensis (F) – rare, depressed (can be as young as early Aptian). |  |
| 15830’ – 15860’ | LAD Schuleridea acuminata (O) FAD Choffatella decipiens (F) | Valanginian |
| 16190’ – 16220’ | FAD Eponides caracolla (F) | No older than Valanginian |
| 16280’ – 16310’ | LAD Sigmomorphina (Ellisina) spatula (F) – rare | No younger than Berriasian |
| 16550’ – 16580’ | LAD Calpionellid bearing wackestone | (no younger than early Valanginian) |

| 16910’ – 16940’ | LAD Reinholdella A (F) break – consistent through 17060’. | No younger than Berriasian [Cotton Valley/Bossier equivalent] |
| --- | --- | --- |
| 17000’ – 17030’ | FAD Lenticulina nodosa (F) | No older than basal Berriasian |
| 17090’ – 17120’ | 17090’-18060’ break of consistent Sigmomorphina (Ellisina) spatula (F) |  |
| 17180’ – 17210’ | LAD Pentaschuleridea pentagonalis (F) FAD Calpionellid bearing wackestone | Tithonian (No older than Lt. Tithonian) |
| 18020’ – 18040’ | FAD Sigmomorphina (Ellisina) spatula (F) |  |
| 18420’ – 18440’ | LAD Epistomina uhligi (F) – sporadic occurrences | Early Tithonian or older [Haynesville equivalent?] |
| 18820’ – 18840’ | LAD Everticyclammina virguliana (F) – rare/depressed (can be as young as Valanginian). |  |
| 19200’ – 19230’ | Well developed oolitic grainstone. | Oxfordian – by correlation [Smackover] (based on lithologic character) |
| 20370’ – 20400’ | Blue-green algal “boundstone” (fenestral matrix) dominates interval 20370’-20820’ – indicates microbial bioherm development. |  |
| 21090’ – 21120’ | Consistent dark calcareous marine shale and argillaceous lime mudstone, pyrite indicative of poorly oxygenated conditions. | [Lower Smackover/ “Brown Dense” equivalent?] (based on lithologic character) |
| 21960’ – 21990’ | Consistent tidal flat anhydrites and laminated anhydritic dolomite, with mix of higher energy bioclastic and oolitic grainstone shoal deposits. |  |
| 22690’ – 22690’ | Red claystone – first indication of non-marine, continental deposits. |  |
| 22750’ – 22780’ | Coarse, red sand – non-marine, continental. | E. Oxfordian – by correlation [Norphlet] (based on lithologic character) |
| 23080’ – 23085’ | Last sample examined. |  |

3. SUMMARY OF PALEOENVIRONMENTAL INTERPRETATIONS

Paleoenvironmental interpretations of analyzed samples are summarized below, by well. Interpretations were based on dominant lithologic characteristics combined with the occurrence of any environmentally diagnostic biogenic material, if present. The key to environment codes is found in Appendix 2.

| Depth Interval (feet) | Environment Code | Paleoenvironment |
| --- | --- | --- |
| 7976.5' - 8000' | OH/RL | High energy, oolitic shoal intermixed with restricted, low-energy subtidal, carbonate dominated |
| 8000' - 8030' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 8030' - 8060' | TE/SH | Evaporitic tidal flat intermixed with high energy, bioclastic shoal, carbonate dominated |
| 8060' - 8180' | RL/TE | Shallow, low-energy, restricted subtidal intermixed with evaporitic tidal flat, carbonate dominated |
| 8180' - 8210' | RL/BH | Shallow, low-energy, restricted subtidal intermixed with shallow, moderate to high-energy biostromal, carbonate dominated |
| 8210' - 8270' | TE/BH | Evaporitic tidal flat intermixed with shallow, moderate to high-energy biostromal, carbonate dominated |
| 8270' - 8420' | TE | Evaporitic tidal flat, carbonate dominated |
| 8420' - 8510' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 8510' - 8660' | TE/RL | Evaporitic tidal flat intermixed with shallow, low-energy, restricted subtidal, carbonate dominated |
| 8660' - 8690' | RL/OH | Shallow, low-energy, restricted subtidal intermixed with high energy, oolitic shoal, carbonate dominated |
| 8690' - 8720' | TE | Evaporitic tidal flat, carbonate dominated |
| 8720' - 8750' | RL/OH | Shallow, low-energy, restricted subtidal intermixed with high energy, oolitic shoal, carbonate dominated |
| 8750' - 8780' | RL/TE | Shallow, low-energy, restricted subtidal intermixed with evaporitic tidal flat, carbonate dominated |
| 8780' - 8810' | OH | High energy, oolitic shoal, carbonate dominated |
| 8810' - 8870' | TE/OH | Evaporitic tidal flat intermixed with high energy, oolitic shoal, carbonate dominated |
| 8870' - 8960' | RL/OH | Shallow, low-energy, restricted subtidal intermixed with high energy, oolitic shoal, carbonate dominated |
| 8960' - 8990' | RL/TE | Shallow, low-energy, restricted subtidal intermixed with evaporitic tidal flat, carbonate dominated |
| 8990' - 9020' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 9020' - 9050' | RL/TE | Shallow, low-energy, restricted subtidal intermixed with evaporitic tidal flat, carbonate dominated |
| 9050' - 9230' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 9230' - 9260' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 9260' - 9290' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 9290' - 9320' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 9320' - 9350' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 9350' - 9440' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 9440' - 9470' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 9470' - 9500' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 9500' - 9560' | RL/OH | Shallow, low-energy, restricted subtidal intermixed with high energy, oolitic shoal, carbonate dominated |
| 9560' - 9590' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 9590' - 9650' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 9650' - 10220' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 10220' - 10430' | OL | Shallow, low-energy, open circulation, carbonate dominated |
| 10430' - 11000' | RL/TE | Restricted, low-energy subtidal intermixed with evaporitic tidal flat, carbonate dominated |
| 11000' - 11150' | TE/SH | Evaporitic tidal flat intermixed with higher energy shoal, carbonate dominated |
| 11150' - 11210' | RL/TE | Shallow, low-energy, restricted subtidal intermixed with evaporitic tidal flat, carbonate dominated |
| 11210' - 11600' | OL-RL | Shallow, low-energy, open to restricted circulation, carbonate dominated |
| 11600' - 11630' | SH | Shallow, high energy, bioclastic shoal, carbonate dominated |
| 11630' - 11840' | OL/BH | Shallow, low-energy, open circulation intermixed with higher energy biostromal, carbonate dominated |
| 11840' - 11960' | OL | Shallow, low-energy, open circulation, carbonate dominated |
| 11960' - 12110' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 12110' - 12140' | OL-RL | Shallow, low-energy, open to restricted circulation, carbonate dominated |
| 12140' - 12200' | OL | Shallow, low-energy, open circulation, carbonate dominated |
| 12200' - 12230' | OL/OH | Shallow, low-energy, open circulation intermixed with high-energy, oolitic shoal, carbonate dominated |
| 12230' - 12500' | OL/SH | Shallow, low-energy, open circulation intermixed with high-energy, bioclastic shoal, carbonate dominated |
| 12500' - 12590' | OL | Shallow, low-energy, open circulation, carbonate dominated |
| 12590' - 12650' | OL/SH | Shallow, low-energy, open circulation intermixed with high-energy, bioclastic shoal, carbonate dominated |
| 12650' - 12770' | OL | Shallow, low-energy, open circulation, carbonate dominated |
| 12770' - 12830' | OL/SH | Shallow, low-energy, open circulation intermixed with high-energy, bioclastic shoal, carbonate dominated |
| 12830' - 13130' | OL | Shallow, low-energy, open circulation, carbonate dominated |
| 13130' - 13190' | OL/SH | Shallow, low-energy, open circulation intermixed with high-energy, bioclastic shoal, carbonate dominated |
| 13190' - 13490' | OL | Shallow, low-energy, open circulation, carbonate dominated |
| 13490' - 13610' | OL/SH | Shallow, low-energy, open circulation intermixed with high-energy, bioclastic shoal, carbonate dominated |
| 13610' - 13850' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 13850' - 14420' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 14420' - 15200' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 15200' - 15380' | RL/OH | Shallow, low-energy, restricted subtidal intermixed with high energy, oolitic shoal, carbonate dominated |
| 15380' - 15440' | OH | High energy, oolitic shoal, carbonate dominated |
| 15440' - 15920' | IN | Inner shelf, terrigenous dominated |
| 15920' - 15980' | IN/OL | Inner shelf terrigenous intermixed with shallow, low-energy, open carbonates |
| 15980' - 16130' | IN | Inner shelf, terrigenous dominated |
| 16130' - 16370' | IN/RL | Inner shelf terrigenous intermixed with shallow, low-energy, restricted carbonates |
| 16370' - 16490' | IN | Inner shelf, terrigenous dominated |
| 16490' - 16910' | OL | Shallow, low-energy, open circulation, carbonate dominated |
| 16910' - 17090' | IN-r | Inner shelf, restricted circulation, terrigenous dominated |
| 17090' - 17900' | RL/IN-r | Shallow, low-energy, restricted subtidal carbonates intermixed with restricted, inner shelf terrigenous |
| 17900' - 18060' | IN-r | Inner shelf, restricted circulation, terrigenous dominated |
| 18060' - 18080' | SH/IN-r | Shallow, high-energy, bioclastic shoal carbonates intermixed with restricted, inner shelf terrigenous |
| 18080' - 18240' | OH/IN-r | Shallow, high-energy, oolitic shoal carbonates intermixed with restricted, inner shelf terrigenous |
| 18240' - 18280' | MM/OH | Marginal marine terrigenous intermixed with high energy, oolitic shoal carbonates |
| 18280' - 18300' | TE/MM | Evaporitic tidal flat intermixed with marginal marine, mixed carbonate/terrigenous |
| 18300' - 18380' | MM/OH | Marginal marine terrigenous intermixed with high energy, oolitic shoal carbonates |
| 18380' - 18420' | TE/MM | Evaporitic tidal flat intermixed with marginal marine, mixed carbonate/terrigenous |
| 18420' - 18460' | IN-r | Inner shelf, restricted circulation, terrigenous dominated |
| 18460' - 18540' | TE/MM | Evaporitic tidal flat intermixed with marginal marine, mixed carbonate/terrigenous |
| 18540' - 18580' | TE/OH | Evaporitic tidal flat intermixed with high energy, oolitic shoal, carbonate dominated |
| 18580' - 18620' | MM/OH | Marginal marine terrigenous intermixed with high energy, oolitic shoal carbonates |
| 18620' - 18720' | MM | Marginal marine, terrigenous dominated |
| 18720' - 18740' | TE/MM | Evaporitic tidal flat intermixed with marginal marine, mixed carbonate/terrigenous |
| 18740' - 18780' | IN-r | Inner shelf, restricted circulation, terrigenous dominated |
| 18780' - 18800' | IN-r/RL | Restricted, inner shelf terrigenous intermixed with shallow, restricted, low-energy carbonates |
| 18800' - 18840' | IN-r | Inner shelf, restricted circulation, terrigenous dominated |
| 18840' - 18960' | IN-r/TE | Restricted, inner shelf terrigenous intermixed with evaporitic tidal flat carbonates |
| 18960' - 19100' | MM | Marginal marine, terrigenous dominated |
| 19100' - 19180' | TE/MM | Evaporitic tidal flat intermixed with marginal marine, mixed carbonate/terrigenous |
| 19180' - 19200' | TE/OH | Evaporitic tidal flat intermixed with high energy, oolitic shoal, carbonate dominated |
| 19200' - 19380' | OH | High energy, oolitic shoal, carbonate dominated |
| 19380' - 19560' | OH/RL | High energy, oolitic shoal intermixed with restricted, low-energy subtidal, carbonate dominated |
| 19560' - 19650' | OH | High energy, oolitic shoal, carbonate dominated |
| 19650' - 19740' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 19740' - 19860' | TE/SH | Evaporitic tidal flat intermixed with high energy, bioclastic shoal, carbonate dominated |
| 19860' - 19920' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 19920' - 19980' | RL/TF | Shallow, low-energy, restricted subtidal intermixed with tidal flat, carbonate dominated |
| 19980' - 20100' | RL | Shallow, low-energy, restricted subtidal, carbonate dominated |
| 20100' - 20370' | RL/OH | Shallow, low-energy, restricted subtidal intermixed with high energy, oolitic shoal, carbonate dominated |
| 20370' - 20820' | RL/BH | Shallow, low-energy, restricted circulation intermixed with higher energy biostromal, carbonate dominated |
| 20820' - 21090' | RL/SH | Shallow, low-energy, restricted subtidal intermixed with high energy, bioclastic shoal, carbonate dominated |
| 21090' - 21960' | IN-r | Inner shelf, restricted circulation, terrigenous dominated |
| 21960' - 22050' | IN-r/SH | Restricted, inner shelf terrigenous intermixed with high-energy, bioclastic shoal carbonates |
| 22050' - 22110' | TE | Evaporitic tidal flat, carbonate dominated |
| 22110' - 22200' | TE/OH | Evaporitic tidal flat intermixed with high energy, oolitic shoal, carbonate dominated |
| 22200' - 22260' | TF/SH | Evaporitic tidal flat intermixed with high energy, bioclastic shoal, carbonate dominated |
| 22260' - 22330' | TF/RL | Tidal flat intermixed with shallow, low-energy, restricted subtidal, carbonate dominated |
| 22330' - 22690' | TE | Evaporitic tidal flat, carbonate dominated |
| 22690' - 22750' | TE/NM | Evaporitic tidal flat carbonates intermixed with non-marine terrigenous |
| 22750' - 22780' | NM/TE | Non-marine terrigenous intermixed with evaporitic tidal flat carbonates |
| 22780' - 22840' | NM | Non-marine, terrigenous dominated |
| 22840' - 22900' | TE/NM | Evaporitic tidal flat carbonates intermixed with non-marine terrigenous |
| 22900' - 22990' | NM | Non-marine, terrigenous dominated |
| 22990' - 23085' | NM/TE | Non-marine terrigenous intermixed with evaporitic tidal flat carbonates |

APPENDIX 1: Paleoenvironmental Zone Definitions

A. Carbonate Paleoenvironmental Zones

The following paleoenvironmental classification is used for stratigraphic sequences that are principally comprised of carbonates deposited in rimmed platform or carbonate ramp settings.

| Code | Description |
| --- | --- |
| TE | Evaporitic Tidal Flat: supratidal flats (sabkha), marginal marine in an evaporative setting. |
| TF | Tidal Flat: supratidal, intertidal, and subtidal elements of carbonate tidal flats. |
| RL | Shallow Shelf, Low Energy, Restricted: subtidal to intertidal areas, between tidal flat and open shelf, restricted circulation. |
| OL | Shallow Shelf, Low Energy, Open: inner shelf ramp, or lagoon behind higher energy shoals or biostromes, open circulation. |
| SH | Shallow Shelf, High-Energy, Bioclastic: carbonate shoals, beaches, reef-associated sands. |
| OH | Shallow Shelf, High-Energy, Oolitic: ooid shoals, banks, and bars over which currents freely ebb and flow. |
| BH | Shallow Shelf, High-Energy, Biostromal: shallow reefs, skeletal build-ups, and associated flank deposits. |
| OS | Middle Shelf, Open: moderately deep open shelf, good circulation, between shallow platform and shelf basin. |
| OB | Deep Shelf, Open: deep intra-shelf basin or outer shelf ramp; good circulation, well oxygenated. |
| RB | Restricted Shelf/Basin: closed, moderate to deep intra-shelf basin, restricted circulation, dysaerobic to anoxic conditions. |
| EB | Evaporitic Basin: enclosed intra-shelf basin; anoxic and evaporitic conditions. |
| SL | Forereef Slope: relatively steep slope in front of shelf margin, dominated by bioclastic debris, gradational with deep marine. |
| DB | Deep Basin: open marine, deepwater basin, seaward of platform or shelf margin. |

Figure 1: Environmental codes for carbonate paleoenvironmental interpretations.

B. Terrigenous Paleoenvironmental Zones

The following paleoenvironmental classification is used for stratigraphic sequences that are principally comprised of terrigenous sediments deposited on a typical continental shelf-slope system.

| Code | Description |
| --- | --- |
| NM | Non-Marine: continental, supratidal, delta plain |
| MM | Marginal Marine: brackish bay, lagoon, tidal, littoral |
| IN | Inner Neritic: shallow marine, shelf, proximal delta, 0-60’ (0-20m) |
| MN | Middle Neritic: middle shelf, marine, distal delta, 60-300’ (20-100m) |
| ON | Outer Neritic: deep shelf, marine, 300-600’ (100-200m) |
| UB | Upper Bathyal: oceanic, upper continental slope, 600-1500’ (200-500m) |
| MB | Middle Bathyal: oceanic, middle slope, 1500-3000’ (500-1000m) |
| LB | Lower Bathyal: oceanic, deep slope, 3000-6000’ (1000-2000m) |
| AB | Abyssal: deep, oceanic basin, continental rise, fans, 6000’+ (2000m+) |

Figure 2: Environmental codes for terrigenous paleoenvironmental interpretations.

C. Mixed Zones

Combinations of paleoenvironmental zones may be indicated within a sample. The following coding convention is used in those cases:

AA-BB – environments gradational from one to another are noted as a pair of zone codes separated by “-“. For example, “OL-OS” is interpreted as “shallow to middle low energy shelf, carbonate dominated”.

AA/BB – Intervals interpreted as predominantly one principal environment with an intermittent secondary environment are noted as a pair of zone codes separated by “/”.  For example, “OL/SH” may be interpreted as “shallow low energy, intermixed with high energy shoals, carbonate dominated”.

Mixed carbonate and terrigenous strata will use similar notations as those above, but will be noted as mixed. For example, “OS/MN” may be interpreted as “low energy, middle shelf, mixed carbonate/terrigenous”.

D. Approximate Terrigenous/Carbonate Zone Equivalents

| Terrigenous Environment | Water depth | Carbonate Environment |
| --- | --- | --- |
| NM | Continental | TE, TF |
| MM | Littoral | TF, RL, OH, SH |
| IN | 0-60’ (0-20m) | OL, RL, SH, OH, BH, EB |
| MN | 60-300’ (20-100m) | OS, RB, EB |
| ON | 300-600’ (100-200m) | OB, RB |
| UB | 600-1500’ (200-500m) | SL |
| MB | 1500-3000’ (500-1000m) | SL, DB |
| LB | 3000-6000’ (1000-2000m) | DB |
| AB | 6000’+ (2000m+) | DB |

APPENDIX 2: Key for Relative Abundance Abbreviations

The following key explains the relative abundance designations for all observed fossil taxa noted in Addendum 1 (Companion Excel File with datasheets Appendix 3 & 4).

| Code |  | Numeric range |
| --- | --- | --- |
| R | Rare | 1-2 specimens |
| F | Few | >2-10 specimens |
| S | Several | >10-20 specimens |
| N | Numerous | >20-50 specimens |
| C | Common | >50-100 specimens |
| A | Abundant | >100-500 specimens |
| D | Dominant | >500 specimens |

Micropaleontology Analysis: Russell E. Hinote

Technical Editor: Nancy L. Engelhardt-Moore